HMoE → FPGA/HLS
What We Have Done So Far

Simple explanation of the repository, our first hardware block, results, gaps, and next steps

# 0. The one-sentence summary

The original repository implements a complete Hierarchical Mixture-of-Experts (HMoE) machine-learning model in PyTorch. We have NOT converted the whole model to hardware yet. We isolated the MoE gating/combination idea, wrote a small C++/Vitis HLS hardware-oriented version of that part, simulated it, synthesized it for a VU13P FPGA, and obtained an initial hardware resource/timing baseline.

# 1. What did the repository do?

The repository is a complete software ML system. At a high level, data goes through several neural-network components, including graph/GCN/GNN-related processing, MLP-related processing, and a hierarchical MoE mechanism.

The important HMoE idea we found is:

There are 3 top-level experts in the default hierarchical component: gnn7, pseudo_alone_w/o_gnn7, and output_mlp.

The model creates a gate value for each expert.

The gate is produced by a linear layer followed by softmax.

Each expert produces an output.

The expert outputs are multiplied by their gate values and added together.

Important: in the default top-level hierarchical configuration, there are 3 experts and top_k=3. Therefore all 3 experts are selected; the softmax decides how much each expert contributes. This is different from a sparse top-1/top-2 MoE where only a subset of experts is executed.

# 2. What part of the repository did we try to do?

We deliberately did NOT start with GNN/GCN/GAT processing or the complete MLP pipeline. The first goal was to understand and implement the MoE routing/gating mechanism as a standalone hardware block.

The block we extracted conceptually is:

320-element hidden vector → Linear(320→3) → 3 logits → Softmax → 3 gates
3 expert outputs + 3 gates → weighted sum → final output

# 3. What exactly did we build?

We created a separate HLS project at ~/HMoE/HLS_HMoE so that the original repository is not modified.

Files:

src/moe.h — dimensions and function declaration.

src/moe.cpp — HLS implementation.

tb/moe_tb.cpp — C simulation/testbench.

scripts/run_hls.tcl — Vitis HLS project, FPGA target, clock, simulation and synthesis commands.

The current implementation takes:

hidden[320]

gate_weight[3][320]

gate_bias[3]

expert_out[3][1]

and produces:

gates[3]

output[1]

# 4. What happens inside our C++ code?

For each of the 3 experts, calculate a linear-layer result: bias + sum(weight × hidden).

Find the largest logit for numerical stability.

Calculate exp(logit - max).

Add the exponentials.

Divide each exponential by the total to obtain softmax gates.

Multiply every expert output by its gate.

Add the weighted expert outputs to obtain the final output.

This matches the basic mathematical operation of the repository's top-level gating and weighted-combination stage, but our hardware block currently receives expert outputs as inputs instead of implementing the experts themselves.

# 5. What is different between the repository and our code?

| Aspect | Repository | Our current HLS block |
| --- | --- | --- |
| Purpose | Complete HMoE ML model | Standalone MoE gating/combination baseline |
| Framework | PyTorch/Python | C++ + Vitis HLS |
| Experts | Actual neural-network expert modules | Expert outputs are supplied as inputs |
| Gating | Linear + top-k + softmax | Linear + softmax |
| Default top-level K | 3 of 3 | 3 of 3 |
| GNN/GCN | Part of the model flow | Not implemented |
| MLP | Part of the model flow | Not implemented |
| Weights/data | Real trained model weights/data | Synthetic deterministic test values |
| Precision | PyTorch floating-point behavior | float32 HLS baseline |
| Training/load balancing | Includes training-related loss machinery | Not implemented; inference path only |
| Hardware | Software execution | Synthesized FPGA hardware |

# 6. If we remove the MoE part from the repository and add ours, will the repository still function?

No — not yet.

Our current block is not a drop-in replacement for the repository's MoE. It only reproduces the mathematical gating/weighted-sum stage for one simplified output dimension. The original repository still needs its real expert computations, real tensors, correct dimensions, model weights, and surrounding data flow.

For the end goal, the hardware block eventually needs to accept the same relevant data/weights as the software implementation and produce numerically equivalent results (within an agreed tolerance). Only then could it replace that part of the software flow.

# 7. How generic is our current implementation?

Conceptually it is generic, but the current code is parameterized only weakly because NUM_EXPERTS=3, TOTAL_HIDDEN=320, TOP_K=3, and OUT_DIM=1 are compile-time constants.

It can be generalized to:

different hidden-vector sizes

different numbers of experts

different output dimensions

different K values

different numerical representations (for example fixed-point later)

However, changing K is not enough by itself. A truly sparse MoE implementation must also decide whether only selected experts are executed, how expert dispatch/gather works, and how outputs are buffered.

# 8. What do the reports say?

Target and performance:

FPGA target: xcvu13p_civ-figd2104-3-e

Clock constraint: 10 ns = 100 MHz

Estimated clock period: 7.140 ns

Estimated Fmax: 140.06 MHz

Latency: 1029 cycles

Latency at 100 MHz: 10.290 µs

Pipeline type: no top-level pipeline

Resource utilization:

| Resource | Used | Available | Approx. utilization |
| --- | --- | --- | --- |
| BRAM_18K | 0 | 5376 | 0% |
| DSP | 14 | 12288 | ~0% |
| FF | 1720 | 3456000 | ~0% |
| LUT | 2314 | 1728000 | ~0% |
| URAM | 0 | 1280 | 0% |

# 9. What is consuming the hardware?

The report shows that floating-point operations dominate the small baseline. In particular, the softmax path (exponential, division, floating-point arithmetic) is expensive compared with the simple control logic.

Floating-point add/sub instance: 2 DSP, 177 FF, 233 LUT

Floating-point multiply instance: 3 DSP, 128 FF, 78 LUT

Softmax-related pipeline: 9 DSP, 657 FF, 1122 LUT

This is useful because it gives us a real baseline against which future FPGA-friendly softmax/fixed-point optimizations can be compared.

# 10. Does this show signs of an actual hardware accelerator?

Yes, but it is an early hardware kernel/block, not yet a complete accelerator.

Vitis HLS successfully converted C++ into synthesizable RTL.

The design was targeted to a real FPGA part.

A clock constraint was applied.

The tool estimated a maximum frequency.

The tool generated real FPGA resource estimates: LUT, FF, DSP, BRAM, URAM.

The latency is reported in hardware clock cycles.

What it does NOT prove yet: that the hardware accelerates the actual repository workload. We have not yet connected real trained weights and real model activations, and we have not compared end-to-end execution time against PyTorch.

# 11. What more needs to be done?

A sensible progression is:

1. Validate against PyTorch: Export real repository gate weights, a real hidden vector, and real expert outputs. Compare Python and HLS results.

2. Make dimensions/configuration faithful: Confirm the exact runtime/default dimensions and output shape used by the selected repository configuration.

3. Implement real expert-side computation where useful: Decide whether GNN/MLP computation should remain software or be moved into FPGA.

4. Improve interfaces/data movement: Design how CPU/DDR/DMA/BRAM/streaming hardware feeds the accelerator.

5. Optimize softmax: Investigate fixed-point, lookup-table, approximation, or other FPGA-friendly alternatives.

6. Implement true sparse routing if desired: If the research goal is sparse MoE, route only selected experts instead of computing every expert.

7. Integrate with the repository: Replace the corresponding software stage and verify functional equivalence.

8. Benchmark: Compare latency, throughput, LUT, FF, DSP, BRAM, power if available, and software baseline.

# 12. Generic MoE vs Repository HMoE vs Our HLS block

| Stage | Generic MoE | Repository HMoE | Our current HLS |
| --- | --- | --- | --- |
| Input | Hidden/features | Hierarchical hidden representation | 320-element hidden vector |
| Gate | Linear → logits | gate_linear → 3 logits | Linear 320→3 |
| Routing | Top-K | Top-3 of 3 in default top-level path | All 3 |
| Normalize | Softmax | Softmax | Softmax |
| Expert compute | Selected experts | Actual gnn7/pseudo/output_mlp paths | Not included; outputs supplied |
| Combine | Σ gate × expert output | Σ gate × expert output | Σ gate × supplied output |
| Output | Combined tensor | Model output passed onward | 1-element output |

# 13. The most important conceptual distinction

There are two separate questions:

A. Can we implement the MoE mathematics in hardware? Yes. We have demonstrated the first small version and synthesized it.

B. Have we accelerated the repository's complete HMoE model? No. That is the larger project still ahead.

# 14. Current status

| Task | Status |
| --- | --- |
| Understand repository's high-level HMoE routing | DONE |
| Isolate top-level MoE gating/combination | DONE |
| Write standalone HLS C++ baseline | DONE |
| C simulation | DONE — PASS |
| FPGA synthesis | DONE — PASS |
| Obtain PPA/timing baseline | DONE |
| Use real trained repository weights/data | NOT DONE |
| Implement actual expert computation | NOT DONE |
| Integrate with repository | NOT DONE |
| Demonstrate acceleration vs software | NOT DONE |

# 15. Bottom line

We have successfully built and synthesized the FIRST hardware piece of the project: a standalone FPGA/HLS implementation of the MoE gate calculation and weighted expert-output combination. It is a legitimate synthesized hardware block and gives us a measurable baseline (1029 cycles, 14 DSP, 1720 FF, 2314 LUT, 0 BRAM, estimated 140.06 MHz).

But it should be called an 'MoE gating/combination hardware baseline', not a 'complete HMoE accelerator'. The next scientifically important step is to feed it real repository data/weights and prove that it reproduces the software MoE output. After that we can decide which expensive stages — expert computation, routing, softmax, and/or data movement — are worth moving into FPGA hardware.