HMoE Output-MoE Hardware Evaluation

Software, Architecture, Verification, and PPA Results

# 1. Software Baseline

The software reference was derived from the repository model implementation and trained checkpoint. The output-MoE uses a 128-dimensional h7 input, four experts, k=4 gating, and five PPA prediction objectives. Since k equals the number of experts, all four experts are selected and their outputs are combined using softmax gate weights.

Each expert contains five independent MLP heads. Each head is 128 → 32 → 16 → 8 → 1, with ELU after the first three layers and a linear final layer. The software reference evaluates all four experts and performs a gate-weighted sum for each objective.

# 2. Common Model Parameters

| Parameter | Value |
| --- | --- |
| Input h7 dimension | 128 |
| Experts | 4 |
| MoE k | 4 |
| Objectives | 5 |
| MLP head | 128 → 32 → 16 → 8 → 1 |
| Activation | ELU after first 3 layers |
| Final layer | Linear |
| Objectives | perf, util-LUT, util-FF, util-DSP, util-BRAM |
| HLS target | xcvu13p_CIV-figd2104-3-e |
| Clock | 10 ns / 100 MHz |
| Test samples | 10 |

# 3. Run 9 — Sequential Hardware Baseline

Run 9 uses one reusable expert engine. The four experts execute sequentially as E0 → E1 → E2 → E3. Within each expert, the five objective heads also execute sequentially. This emphasizes hardware resource sharing.

The design therefore maps four logical experts onto one physical reusable computation engine, followed by gate-weighted accumulation.

# 4. Run 11 — Parallel Hardware Baseline

Run 11 keeps the same trained parameters and numerical algorithm as Run 9, but changes the hardware mapping. Engine A processes E0 → E2 and Engine B processes E1 → E3. HLS DATAFLOW allows the two engines to operate concurrently.

Each engine still processes its assigned experts sequentially, and the five objective heads inside each expert remain sequential. Thus the added parallelism is expert-level, not objective-head-level.

The engine outputs are remapped to E0, E1, E2, E3 before the final gate-weighted sum, so the algorithmic output calculation remains unchanged.

# 5. Architecture Comparison

The key difference is physical replication. Run 9 maps four logical experts onto one reusable engine. Run 11 maps them onto two engines, allowing two experts to be processed concurrently. No trained weights, biases, MLP dimensions, gate formulation, or final weighted-sum algorithm were changed.

# 6. Functional Verification

The HLS testbench reads 10 samples of the 128-dimensional h7 input, reference gate values, and reference output data. For each sample it calls output_moe(), checks all four gates and their sum, and prints all five objective outputs.

The testbench is a verification wrapper; it does not implement the MoE algorithm. Expected values originate from the software reference/golden data. The tolerance is 1e-4.

Run 9 corrected software/HLS comparison showed gate and objective differences within floating-point tolerance. Run 11 C simulation passed its functional checks, but retains a previously observed Sample 6 perf discrepancy as an explicitly ignored warning. This should be reported as a known limitation rather than hidden as a clean verification result.

# 7. Synthesis and PPA Results

| Metric | Run 9 | Run 11 | Change |
| --- | --- | --- | --- |
| Latency | 94,685 cycles | 46,379 cycles | −51.0% |
| Time @ 100 MHz | 946.85 µs | 463.79 µs | −51.0% |
| BRAM | 181 | 204 | +12.7% |
| DSP | 196 | 487 | +148.5% |
| FF | 51,979 | 49,834 | −4.1% |
| LUT | 38,920 | 152,325 | +291.2% |

Run 11 approximately halves latency while substantially increasing replicated hardware. The strongest resource increases are LUTs and DSPs, illustrating the latency-versus-resource tradeoff created by expert-level parallelism.